Energy Meter Theory: https://www.digikey.com/en/articles/the-basics-of-current-sensors

Current Sensor Exact: ACS773ECB-200B-PFF-T

Mating Housing: page 9 of datasheet

5 segments

Current sensor 3 pole voltage - which one

Bit bumping to fill the byte - 5 sensors across 32 bits?

Adc vs using adc on esp32

Program the adc?

Should i filter the input to adc

What protocol is one wire - serial without a clk?????? - BIT BANGING

Precision necessary for adc to be separate?

Parasitic power

Constraints:

- Integrate energy meter with mating device (requires cutting and extrusion)

- Energy meter in series with TS path

  - Depending on current sensor it will be over CAN or analog

- ESP32 to read current sensor output

- Autotransformer to step down 12 V to 6.5 V

- Include connector to pull data from

- Needs to be logged

  - Turn off tractive system, keep GLV on and pull data from PCB - isolated from rest of car

- Cf and Rf for filtering on Vout

- Cbyp for Vcc filtering

Requirements:

- 3 TS connections:

  - 2: wired in series with tractive system for current sensor

    - Cant be wired mid pack

  - 1: wired to other pole for sensing voltage

- Must be TRACTIVE SYSTEM SIDE OF ISOLATION RELAYS

- 7 GLV connections:

  - 1 - temp sense gnd for HV

  - 2 - temp sense one wire

  - 3 - no connection

  - 4 - CAN L

  - 5 - CAN H

  - 6 - GLV -

  - 7 - GLV +

- Energy meter requirements

  - Current draw: 125 mA

  - CAN rate 500k

  - Access real time energy meter data over CAN

  - Can log various vehicle info to energy meter over CAN (conveinence)

  - ALL CAN IS OPTIONAL

  - CAN IS BIG ENDIAN

Includes terminating resistor

- ENERGY METER TRANSMITTED MESAGES

- 1 energy meter temp sensor PER SEGMENT

  - IN HOTTEST SECTION

INTERFACING LOGISTICS:

- GLV power and energy meter boot

- Begin logging when TS > 60 V

- Stop logging when TS < 60 V

- LED indicators:

DOWNLOADING DATA

- Download cable will supply power, not GLV